CPE301 – SPRING 2019

# Design Assignment X

Student Name: Jeb Marinas

Student #:5003376451

Student Email: marinj4@unlv.nevada.edu

Primary Github address:github.com/jebmarinas

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

void adc_init(void);

void read_adc(void);

volatile int control = 0; //this will control the motor

volatile unsigned int speed; // this will control how was the motor go

volatile unsigned int adc;

void adc_init(void)

{

ADMUX = (0<<REFS1)| // reference selection bit

(1<<REFS0)| //AVCC EXTERNAL CAP AT REF

(1<<ADLAR)| //ADC LEFT ADJUST RESULT

(0<<MUX2)| //ANALOG CHANNEL SELECTION BITS

(0<<MUX1)| //ADC

(0<<MUX0);

ADCSRA = (1<<ADEN)| // ADC ENABLE

(0<<ADSC)| // ADC START CONVERSION

(0<<ADATE)| // ADC AUTO TRIGGER ENABLE

(0<<ADIF)| // ADC INTERRUPT ENABLE

(0<<ADIE)|// ADC PRESCALER SELECT BIT

(1<<ADPS2)|

(1<<ADPS1)|

(1<<ADPS0); // SELECT CHANNEL

}

void read_adc(void)

{

unsigned char i = 4;

adc = 0;

while(i--)

{

ADCSRA |=(1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc += ADC;

_delay_ms(50);

}

adc = (adc/8);

}

int main(void)

{

DDRB = 0x0F;//will be used as an output

PORTB = 0x00;// the voltage in it will be 0

TCCR1B = (1 << WGM13)|(1 << WGM12)|(1 << CS11);// THE PRESSCALAR WILL BE AT 8

ICR1 = 62258;

adc_init();

while(1)

{

read_adc();

_delay_ms(30);

}

while(1)

{

PORTD = 0b01100110;

_delay_ms(50);

PORTD = 0b11001100;

_delay_ms(50);

PORTD = 0b10011001;

_delay_ms(50);

PORTD = 0b00110011;

_delay_ms(50);

if ((adc >= 981) && (adc < 1024 ))	//when adc value is at its max

{

control = 1;

_delay_ms(20);

}

else if ((adc < 982) && (adc >= 758)) //will decrease pwm

{

control = 0;

speed = 0x1869;

_delay_ms(50);

}

else if ((adc < 770) && (adc >= 67)) //will decreasing pwm

{

control = 0;

speed = 0x124F;

_delay_ms(100);

}

else

{

PORTD = 0b00000000;//if resistor value is less than 5%

_delay_ms(100);

}

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- #define F_CPU 16000000UL

- #include <avr/io.h>

- #include <util/delay.h>

- #include <avr/interrupt.h>

- void adc_init(void);

- void read_adc(void);

- volatile float adc;

- int check = 0;

- void adc_init(void)

- {

- ADMUX = (0<<REFS1)| // reference selection bit

- (1<<REFS0)| //AVCC EXTERNAL CAP AT REF

- (1<<ADLAR)| //ADC LEFT ADJUST RESULT

- (0<<MUX2)| //ANALOG CHANNEL SELECTION BITS

- (0<<MUX1)| //ADC

- (0<<MUX0);

- ADCSRA = (1<<ADEN)| // ADC ENABLE

- (0<<ADSC)| // ADC START CONVERSION

- (1<<ADATE)| // ADC AUTO TRIGGER ENABLE

- (0<<ADIF)| // ADC INTERRUPT ENABLE

- (0<<ADIE)|// ADC PRESCALER SELECT BIT

- (1<<ADPS2)|

- (1<<ADPS1)|

- (0<<ADPS0); // SELECT CHANNEL

- }

- void read_adc(void)

- {

- unsigned char i = 4;

- adc = 0;

- while(i--)

- {

- ADCSRA |=(1<<ADSC);

- while(ADCSRA & (1<<ADSC));

- adc += ADC;

- _delay_ms(50);

- }

- adc = (adc/8);

- }

- int main(void)

- {

- adc_init(); //Initializing ADC

- PORTC |= 0b00000001;

- DDRD = 0xFF; DDRB = 0xFF;//outputs

- //Configuration of Timer1

- TCCR1A |= (1 << COM1A1) |(1 << COM1B1) |(1 << WGM11);

- TCCR1B |= (1 << WGM13) |(1 << WGM12) |(1 << CS11) |(1 << CS10);

- ICR1 = 62258;

- while(1)

- {

- read_adc();

- _delay_ms(50);

- while(1)

- {

- ADCSRA |= (1<< ADSC);

- while((ADCSRA & (1 << ADIF)) == 0);

- check = ADCH;

- if(check == 0)

- {

- OCR1A = 0;

- _delay_ms(200);

- }

- if(check == 255)

- {

- OCR1A = 500;

- _delay_ms(200);

- }

- }

- }

- }

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT