CPE301 – SPRING 2019

# Design Assignment X

Student Name: Jeb Marinas

Student #:5003376451

Student Email: marinj4@unlv.nevada.edu

Primary Github address:github.com/jebmarinas

Directory:

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328pb

FTDI chip

LM34

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

- #define F_CPU 16000000UL

- #define BAUD 9600

- #include <avr/io.h>

- #include <util/delay.h>

- #include <avr/interrupt.h>

- #include <util/setbaud.h>

- #include <stdio.h>

- volatile uint8_t of;

- volatile float adc_temp;

- void usart_init(void)

- {

- UBRR0H = UBRRH_VALUE; // loads the ubrrh value into high

- UBRR0L = UBRRL_VALUE;// loads the ubrr0l value into low

- UCSR0C = _BV(UCSZ01) | _BV(UCSZ00); // 8-bit data

- UCSR0B = _BV(RXEN0) | _BV(TXEN0);// enable RX and TX

- }

- void set()

- {

- TCCR0A = 0; // normal mode is initiated

- TCCR0B = 0x05;// the prescaler is set to 1024

- TCNT0 = 0x00;// initiates the counter to 0

- TIMSK0 = (1<<TOIE0); // enable interrupt

- sei(); // enable global interrupt

- }

- void usart_tx_string(char * info)

- {

- while((*info != '\0'))

- {

- while( !(UCSR0A & (1 << UDRE0)));

- UDR0 = *info;

- info++;

- }

- }

- ISR(TIMER0_OVF_vect)

- {

- of++; // this will increment the overflow

- }

- void adc_init(void)

- {

- ADMUX = (0<<REFS1)| // reference selection bit

- (1<<REFS0)| //AVCC EXTERNAL CAP AT REF

- (0<<ADLAR)| //ADC LEFT ADJUST RESULT

- (1<<MUX2)| //ANALOG CHANNEL SELECTION BITS

- (0<<MUX1)| //ADC

- (0<<MUX0);

- ADCSRA = (1<<ADEN)| // ADC ENABLE

- (0<<ADSC)| // ADC START CONVERSION

- (0<<ADATE)| // ADC AUTO TRIGGER ENABLE

- (0<<ADIF)| // ADC INTERRUPT ENABLE

- (0<<ADIE)|// ADC PRESCALER SELECT BIT

- (1<<ADPS2)|

- (1<<ADPS1)|

- (1<<ADPS0); // SELECT CHANNEL

- }

- void read_adc(void)

- {

- unsigned char i = 4;

- adc_temp = 0;

- while(i--)

- {

- ADCSRA |=(1<<ADSC);

- while(ADCSRA & (1<<ADSC));

- adc_temp += ADC;

- _delay_ms(50);

- }

- adc_temp = (adc_temp/4);

- }

- int main(void)

- {

- usart_init();

- set();

- adc_init();

- char c[20];

- while(1)

- {

- while(of < 61);

- read_adc();

- snprintf(c,sizeof(c), "%f\r\n", adc_temp);

- usart_tx_string(c);

- usart_tx_string("\n");

- of = 0;

- }

- }

SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=O5QHUHwZO6E

- GITHUB LINK OF THIS DA

https://github.com/jebmarinas/cpe301/tree/master/hw3b

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT